Aim of the Experiment:

Physical Computation of different Embedded System Projects using push-button switch interfacing with Raspberry Pi and LEDs.

Objective:

- Implementation of mini pedestrian crossing system (simple traffic light control system) using multiple LEDs without push-button.

- Study the effect of Multithreading concept on processing time of an embedded system using Micro-python script (e.g. Comparing the processing time to calculate the square and cube of a series of numbers with and without multithreading concept).

- Implementation of mini pedestrian crossing system with proper control (Puffin Crossing System) based on the concept of multi-threading using multiple LEDs with a push-button and a buzzer.

- Implementation of Single player based Simple Reaction Timing Game (Mental Chronometry) based on the concept of Interrupt Requests, or IRQs using a push-button.

- Implementation of Two player based Simple Reaction Timing Game (Mental Chronometry) based on the concept of Interrupt Requests, or IRQs using a push- button.

- Implementation of changing the LED flashing rate using pushbutton as external interrupts with external and internal Pull-up resistors.

- Implementation of a Digital Dice Game using LEDs and Push-buttons.

- Implementation of Lucky Day of the Week using LEDs and Push-buttons.

Components/Equipment/items Required:

| Sl No. | Name of the Component/Equipment | Specification | Quantity |
| --- | --- | --- | --- |
| 1 | Raspberry Pi Pico | RP2040 microcontroller chip, 125MHz | 1 |
| 2 | Raspberry Pi Pico cable | USB Type A to Micro-B | 1 |
| 3 | Resistors (carbon type) | ¼ watt (330 Ω) | 10 |
| 4 | LED | 3mm, Red | 8 |
| 5 | Tactile Push Button Switches | 6 x 6 x 6 mm | 2 |
| 6 | Breadboard | 840 Tie points | 1 |
| 7 | Jumper Wire | --------------------------- | As per requirement |

Circuit/Schematic Diagram:

Objective 1

(Figure 1: Circuit diagram for implementation of mini pedestrian crossing system.)

Objective 3

(Figure 2: Circuit diagram for implementation of Puffin crossing system.)

Objective 4

(Figure 3: Circuit diagram for implementation of Single player based Simple Reaction Timing Game.)

Objective 5

(Figure 4: Circuit diagram for implementation of Two player based Simple Reaction Timing Game.)

Objective 6

(Figure 5: Circuit diagram for implementation of changing the LED flashing rate using pushbutton with Pull-up resistors.)

Objective 7

(Figure 6: Circuit diagram for implementation of a Digital Dice Game using LEDs and Push-buttons.)

Objective 8

(Figure 7: Circuit diagram for implementation of Lucky Day of the Week using LEDs and Push-buttons.)

Observation:

Objective 1

(Figure 8: Simulation based electronic circuit for implementation of mini pedestrian crossing system.)

Figure 9: (Breadboard Schematic representation of an electronic circuit for implementation of mini pedestrian crossing system.)

Figure 10: (Hardware implementation based electronic circuit for implementation of mini pedestrian crossing system)

Objective 2

(Figure 11: Simulation based Comparing the processing time to calculate the square and cube of a series of numbers without multithreading concept.)

(Figure 12: Simulation based Comparing the processing time to calculate the square and cube of a series of numbers without multithreading concept.)

(Figure 13: Simulation based electronic circuit for implementation of Puffin crossing system.)

Figure 14: (Breadboard Schematic representation of an electronic circuit for implementation of Puffin crossing system.)

Figure 15: (Hardware implementation based electronic circuit for implementation of Puffin crossing system)

Objective 4

(Figure 16: Simulation based electronic circuit for implementation of Single player based Simple Reaction Timing Game.)

Figure 17: (Breadboard Schematic

representation of an electronic circuit for implementation of Single player

based Simple Reaction Timing Game.)	Figure 18: (Hardware implementation based electronic circuit for

implementation of Single player based Simple Reaction Timing Game)

(Figure 19: Simulation based electronic circuit for implementation of Two player based Simple Reaction Timing Game.)

Figure 20: (Breadboard Schematic representation of an electronic circuit for implementation of Two player based Simple Reaction Timing Game.)

Figure 21: (Hardware implementation based electronic circuit for implementation of Two player based Simple Reaction Timing Game)

Objective 6

(Figure 22: Simulation based electronic circuit for changing the LED flashing rate using pushbutton with Pull-up resistors.)

Figure 23: (Breadboard Schematic representation of an electronic circuit for changing the LED flashing rate using pushbutton with Pull-up resistors.)

Figure 24: (Hardware implementation based electronic circuit for changing the LED flashing rate using pushbutton with Pull-up resistors.)

Objective 7

Figure 25: (Simulation based electronic circuit for implementation of a Digital Dice Game using LEDs and Push-buttons.)

Figure 26: (Hardware implementation based electronic circuit for implementation of a Digital Dice Game using LEDs and Push-buttons.)

Objective 8

Figure 27: (Simulation based electronic circuit for Lucky Day of the Week using LEDs and Push-buttons.)

Figure 28: (Hardware implementation based electronic circuit for Lucky Day of the Week using LEDs and Push-buttons.)

Codes:

Objective 1

print("Hello, Pi Pico!")

print("This is Experiment - 5 and Objective - 1")

print("Name:Bali Babu Chauhan	; Registration No.:19410121182	") print("Objective : 1 Implementation of mini pedestrian crossing system using multiple LEDs without push-button ")

from machine import Pin

import utime

def glow(bulb,duration=0.1):

bulb.value(1)

utime.sleep(duration)

bulb.value(0)

red=Pin(1,Pin.OUT)

yellow=Pin(2,Pin.OUT)

green=Pin(3,Pin.OUT)

durations=[5,2,7]

button=Pin(4,mode=Pin.IN, pull=Pin.PULL_DOWN)

lights=[red,yellow,green]

while True:

for light,duration in zip(lights,durations):

c=0

while c<duration*10:

c+=1

glow(light)

while button.value()==1:

glow(red,duration=5)

Objective 2

print("Objective : 2	Study the effect of Multithreading concept on processing time of an embedded system using Micro-python script ")

import utime,_uthread

def square(lis):

newList=[]

for i in lis:

newList.append(i*i)

def cube(lis):

newList=[]

for i in lis:

newList.append(i*i*i)

_thread.start_new_thread(clickListener,(button,))

lis=[1,2,3,4,6,7]

t=utime.ticks_us()

square(lis)

cube(lis)

t2=utime.ticks_us()

print('time taken',(t2-t))

Objective 3

print("Objective : 3	Implementation of Puffin Crossing System based on the concept of multi-threading using multiple LEDs with a push-button and a buzzer.")

from machine import Pin

import utime, _thread

isPressed=0

def clickListener(button):

global isPressed

while True:

if button.value()==0: isPressed=0

else: isPressed=1

def glow(bulb,duration=0.1):

bulb.value(1)

utime.sleep(duration)

bulb.value(0)

red=Pin(1,Pin.OUT)

yellow=Pin(2,Pin.OUT)

green=Pin(3,Pin.OUT)

durations=[5,2,7]

button=Pin(4,mode=Pin.IN, pull=Pin.PULL_DOWN)

lights=[red,yellow,green]

_thread.start_new_thread(clickListener,(button,))

while True:

for light,duration in zip(lights,durations):

c=0

while c<duration*10:

c+=1

glow(light)

while isPressed==1:

glow(red,duration=5)

Objective 4

print("Objective : 4	Implementation of Single player based Simple Reaction Timing Game based on the concept of Interrupt Requests, or IRQs using a push- button.”)

import machine, urandom, utime

led = machine.Pin(1, machine.Pin.OUT)

button = machine. Pin(28, machine.Pin.IN)

def button_press(pin):

button.irq(handler=None)

rection_time = utime.ticks_diff(utime.ticks_ms(), timer_light_off)

print("Your reaction time was " + str(rection_time) + " milliseconds!")

led.value(1)

utime.sleep(urandom.uniform (5, 10))

led.value(0)

timer_light_off = utime.ticks_ms()

button.irq(trigger=machine.Pin.IRQ_RISING, handler=button_press)

Objective 5

print("Objective : 5	Implementation of Two player based Simple Reaction Timing Game based on the concept of Interrupt Requests, or IRQs using a push-button.")

import machine, urandom, utime

led = machine.Pin(1, machine.Pin.OUT)

Lbutton = machine. Pin(28, machine.Pin.IN)

Rbutton = machine. Pin(20, machine.Pin.IN)

def button_press(pin):

Lbutton.irq(handler=None)

Rbutton.irq(handler=None)

rection_time = utime.ticks_diff(utime.ticks_ms(), timer_light_off)

if pin==Lbutton:

print('Left Player Won')

else:

print('Right Player Won')

print("Your reaction time was " + str(rection_time) + " milliseconds!")

led.value(1)

utime.sleep(urandom.uniform (5,10))

led.value(0)

timer_light_off = utime.ticks_ms()

Lbutton.irq(trigger=machine.Pin.IRQ_RISING, handler=button_press)

Rbutton.irq(trigger=machine.Pin.IRQ_RISING, handler=button_press)

Objective 6

print("Objective : 6	Implementation of changing the LED flashing rate using pushbutton as external interrupts with external and internal Pull-up resistors.")

from machine import Pin

import utime, urandom

bulb = machine.Pin(1, machine.Pin.OUT)

Lbutton = machine. Pin(28, machine.Pin.IN)

Rbutton = machine. Pin(20, machine.Pin.IN)

def display(duration=0.1):

bulb.high()

utime.sleep(duration)

bulb.low()

utime.sleep(duration)

c=1

while True:

if Lbutton.value()==1:

c+=1

if Rbutton.value()==1:

c-=1

display(c)

Objective 7

print("Objective : 7	Implementation of a Digital Dice Game using LEDs and Push-buttons.")

from machine import Pin

import utime,urandom

button=Pin(16,Pin.IN)

leds=[Pin(i,Pin.OUT) for i in range(1,8)]

number={1:'4',2:'35',3:'345',4:'1267',5:'12467',6:'123567'}

def glowBulb(strPin,duration=0.1):

for i in strPin:

Pin(int(i),Pin.OUT).value(1)

utime.sleep(duration)

for i in strPin:

Pin(int(i),Pin.OUT).value(0)

def button_press(pin):

button.irq(handler=None)

utime.sleep(2)

for i in range(1,8):

Pin(i,Pin.OUT).value(0)

while True:

button.irq(trigger=Pin.IRQ_RISING, handler=button_press)

glowBulb(number[int(urandom.uniform(1,7))])

Objective 8

print("Objective : 8	Implementation of Lucky Day of the Week using LEDs and Push-buttons.")

from machine import Pin

import utime, urandom

pushButton=Pin(28,Pin.IN)

weekName=['sunday', 'monday', 'tuesday', 'wednesday', 'thursday', 'friday', 'saturday']

weekPins={'sunday':1, 'monday':2, 'tuesday':4, 'wednesday':5, 'thursday':7, 'friday':6, 'saturday':3}

def buttonAction(pin):

global randomNumber

pushButton.irq(handler=None)

print('Your lucky day of the week is',weekName[randomNumber])

utime.sleep(3)

def glowBulb(pinNumber):

Pin(pinNumber,Pin.OUT).value(1)

utime.sleep(0.01)

Pin(pinNumber,Pin.OUT).value(0)

randomNumber=1

while True:

randomNumber=int(urandom.uniform(0,7))

glowBulb(weekPins[weekName[randomNumber]])

pushButton.irq(trigger=Pin.IRQ_RISING, handler=buttonAction)

Conclusion: